#define HwBaseAddress                           0x00200000

#define LcdCmdAddress 0x00500000

void	wait()

{

int		i,dummy,dummy1;

for(i=0;i<10000;i++)

{

dummy++;

dummy1 = dummy;

}

}

void

LCD_cmd_write(unsigned char  Lcmd)

{

unsigned long * volatile pulPtr = (unsigned long *)HwBaseAddress;

unsigned char * Lcd_cmd = (unsigned char *)LcdCmdAddress;

((unsigned char *)pulPtr)[HwPortD] &= 0xfd;

*Lcd_cmd = Lcmd;

((unsigned char *)pulPtr)[HwPortD] |= 0x02;

}

void

LCD_data_write(unsigned char  Ldata)

{

unsigned char * Lcd_data = (unsigned char *)LcdCmdAddress;

//    ((unsigned char *)pulPtr)[HwPortD] &= 0xfd;

*Lcd_data = Ldata;

//    ((unsigned char *)pulPtr)[HwPortD] |= 0x02;

}

unsigned char

LCD_cmd_read(void)

{

unsigned long * volatile pulPtr = (unsigned long *)HwBaseAddress;

unsigned char * Lcd_cmd = (unsigned char *)LcdCmdAddress;

unsigned char	status;

((unsigned char *)pulPtr)[HwPortD] &= 0xfd;

status = *Lcd_cmd;

((unsigned char *)pulPtr)[HwPortD] |= 0x02;

return(status);

}

void set_cur(unsigned int x, unsigned int y)

{

unsigned char	page_addr_set = 0xb0, h_col_addr = 0x10,  l_col_addr = 0x00,x_cur_pos;

x_cur_pos = x*8;

LCD_cmd_write(page_addr_set + y);

LCD_cmd_write(l_col_addr + (unsigned char)(x_cur_pos & 0x0f));

LCD_cmd_write(h_col_addr + (unsigned char)((x_cur_pos >> 4) & 0x0f));

}

void	LCDOn()

{

unsigned char	LCD_on_cmd =0xaf;

LCD_cmd_write(LCD_on_cmd);

}

void	LCDOff()

{

unsigned char	LCD_off_cmd =0xae;

LCD_cmd_write(LCD_off_cmd);

}

void LCD_init(void)

{

unsigned long * volatile pulPtr = (unsigned long *)HwBaseAddress;

unsigned char	power_control = 0x2f,registor_ratio = 0x23,volume_mode = 0x81,volume_reg = 0x25,com_output_mode = 0xc8;

unsigned char   display_start_line = 0x40,status;

((unsigned char *)pulPtr)[HwPortD] |= 0x02;

((unsigned char *)pulPtr)[HwPortD] |= 0x04;

wait();

((unsigned char *)pulPtr)[HwPortD] &= 0xfb;

wait();

((unsigned char *)pulPtr)[HwPortD] |= 0x04;

wait();

while(1)

{

status = LCD_cmd_read();

if((status & 0x80) == 0)  break;

}

LCD_cmd_write(power_control);

LCD_cmd_write(registor_ratio);

LCD_cmd_write(volume_mode);

LCD_cmd_write(volume_reg);

LCD_cmd_write(com_output_mode);

LCD_cmd_write(display_start_line);

Clear_Lcd();

LCDOn();

}

void ClrLine(int x)

{

unsigned char	page_addr_set = 0xb0, h_col_addr = 0x10,  l_col_addr = 0x00;

int i,j;

for(i=2*x;i <2*x+2; i++)

{

LCD_cmd_write(page_addr_set + i);

LCD_cmd_write(l_col_addr);

LCD_cmd_write(h_col_addr);

for(j=0;j<128;j++)

LCD_data_write(0x00);

}

}

void Clear_Lcd(void)

{

ClrLine(0);

ClrLine(1);

ClrLine(2);

ClrLine(3);

}